Name of the Work :-  Development works to the Sri Venkataramana Temple, Sri

Brahma and Parivara Mandira and Sri Kashi Vishwanatha

Temple  at Chantharu  Village,   Udupi Taluk & District.

ABSTRACT

| S.No | Brief Description of the Work | Amount (Rs.) |
| --- | --- | --- |
| 1. 2. 3. 4. | Reconstruction of the Sri Venkataramana Temple:- Granite size stone masonry in C.M 1:6, for foundation over C.C bed of 1:4:8, laterite masonry in 1:6 for superstructure, Copper sheet roofing to Garbha griha and remaining portion with Mangalore tiled roofing over best teak wood rafter, wall plate etc,, best teak wood designed door, Wooden dali (Grill work), plastering to wall surface in C.M 1:6, polished granite slabs flooring and semi polished granite tiled cladding to outer wall surface, providing decorative art work in basement, etc., Reconstruction of the Sri Brahma and Parivara Mandira:- Granite size stone masonry in C.M 1:6, for foundation over C.C bed of 1:4:8, laterite masonry in 1:6 for superstructure, Mangalore tiled roofing over best country wood rafter, wall plate etc,, best country wood designed door, plastering to wall surface in C.M 1:6, polished granite slabs flooring and skirting,semi polished granite tiled cladding to outer wall surface, , providing decorative art work in basement, etc., Renovation to the Mangalore tiled roofing of the Garbha griha, Hebbagilu, Powli of the Sri Kashi Vishwanatha Temple:- Providing new Mangalore tiled roofing by replacing the old deteriorated wall plate, Rafters, reepers and tiles to new ones, wood oiling, fixing of ridge tiles, providing lime mortar bands, etc complete (350.00 Sqmts) Renovation to the Sri Naagalaya and Chitra koota :- Granite size stone masonry for foundation over CC bed 1:4:8, sand filling inside the basement, four line dressed granites stone Adhistana, ‘Sophana’ as per Shilpa Shastra, dressed granite slab flooring, etc., | 20,00,000.00 5,00,000.00 12,00,000.00 3,00,000.00 |

-2-

| Sl.No | Brief Description of the Work | Amount (Rs.) |
| --- | --- | --- |
| 4. 5. 6. 7. | Construction of compound wall (100.00 Mts) Granite size stone masonry for foundation over CC bed 1:4:8,laterite stone masonry wall in C.M 1:6, plastering with C.M 1:6,Colour washing. Providing Dressed Granite Slab paving Proving three line Dressed Granite Slab paving for the Inner Angana of the sri Kashi Vishwanatha Temple. by raising the existing Ground level up to 25 Cms (105.00 Sqmts) Providing Interlock block paving Proving 50 mm thick interlock concrete blocks paving in front of the sri Kashi Vishwanatha Temple. over the sand bed (500.00 Sqmts) Construction of Staff Quarters, Public toilet Block, Welcome Arch Etc | 3,00,000.00 4,00,000.00 4,00,000.00 14,00,000.00 |

Total Amount                           Rs: 65,00,000.00

(Rupees Sixty Five Lakhs  only.)

President /Secretary                                                                  Engineer.

Renovation Committee

-3-

Name of the Work :-  Reconstruction of the Sri Venkataramana Temple

at Chanthar Village,   Udupi Taluk & District.

ABSTRACT  ESTIMATE

| Sl.No | Brief Description of Work | Quantity | Unit | Rate | Amount (Rs) |
| --- | --- | --- | --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. 8. 9. | Earthwork excavation for foundation in all kind of soils including all lead and lift charges. etc., complete. Providing and laying CC bed 1:4:8 for foundation and flooring using 40mm size jelly including curing etc., complete. Providing and constructing granite size stone masonry for Foundation in C.M 1:6,including curing etc., Providing and constructing granite size stone masonry for Basement in C.M 1:6,including curing etc., Providing cement concrete 1:3:6 for basement including cost of all materials and labour and curing. Earth filling to the basement using new approved earth including consolidation of the earth etc., Providing and constructing laterite size stone masonry in CM 1:6, for super structure, including curing, scaffolding etc., complete. Providing Mangalore tiled Roofing over best country wood Rafters wall plate, reepers etc including cost of all materials, ridge tiles lime mortar band etc., complete . Providing Copper sheet roofing over best country wood Rafters wall plate, planks etc including cost of all materials, copper nails, labours etc., complete . | 22.00 4.00 15.00 8.00 1.00 10.00 13.00 70.00 27.00 | Cumt Cumt Cumt Cumt Cumt Cumt Cumt Sqmt Sqmt | 205.00 4457.00 2807.00 3281.00 5017.00 300.00 3144.00 6000.00 28000.00 | 4510.00 17828.00 42105.00 26248.00 5017.00 3000.00 40872.00 420000.00 756000.00 |

-4-

| Sl.No | Brief Description of Work | Quantity | Unit | Rate | Amount (Rs) |
| --- | --- | --- | --- | --- | --- |
| 10. 11. 12. 13. 14. 15. 16. | Providing and fixing best teak wood Grill work (Dali),with frame work and patty including cost of materials, labour, complete as per specifications and design Providing plastering to wall surface in CM 1:6, 20mm thick including scaffolding, curing etc., Providing approved quality polished Granite slabs flooring/skirting, slabs set in CM 1:4, including curing etc., Providing granite tiled dadooing using 10mm thick flamed granite tiles.tiles set in C.M 1:4, including cost of materials, pointing, scaffolding etc., complete. Providing and fixing best Teak wood doors with fully paneled shutters including cost of fittings, polishing etc., complete. Providing Decorative cornice works in Basement etc., as per design. Contingencies. and consultation charges. | 24.00 105.00 25.00 50.00 - - - | Sqmt Sqmt Sqmt Sqmt - - - | 15000.00 250.00 1800.00 1200.00 L.S L.S L.S | 360000.00 26250.00 45000.00 60000.00 40000.00 100000.00 53170.00 |

### Total Amount:         Rs. 20,00,000.00

(Rupees Twenty Lakhs only.)

-5-

Name of the Work :-  Reconstruction of the Sri Brahma and Parivara Mandira

at Chanthar Village,   Udupi Taluk & District.

ABSTRACT  ESTIMATE

| Sl.No | Brief Description of Work | Quantity | Unit | Rate | Amount (Rs) |
| --- | --- | --- | --- | --- | --- |
| 1. 2. 3. 4. 5. 6. 7. 8. 9. | Earthwork excavation for foundation in all kind of soils including all lead and lift charges. etc., complete. Providing and laying CC bed 1:4:8 for foundation and flooring using 40mm size jelly including curing etc., complete. Providing and constructing granite size stone masonry for Foundation in C.M 1:6,including curing etc., Providing and constructing granite size stone masonry for Basement in C.M 1:6,including curing etc., Providing cement concrete 1:3:6 for basement including cost of all materials and labour and curing. Earth filling to the basement using new approved earth including consolidation of the earth etc., Providing and constructing laterite size stone masonry in CM 1:6, for super structure, including curing, scaffolding etc., complete. Providing Mangalore tiled Roofing over best country wood Rafters wall plate, reepers etc including cost of all materials, ridge tiles lime mortar band etc., complete . Providing and fixing best country wood designed doors with fully paneled shutters including cost of fittings, polishing etc., complete. | 14.00 3.00 7.00 6.00 1.00 10.00 10.00 40.00 2.00 | Cumt Cumt Cumt Cumt Cumt Cumt Cumt Sqmt Sqmt | 205.00 4457.00 2807.00 3281.00 5017.00 300.00 3144.00 3000.00 10000.00 | 2870.00 13371.00 19649.00 19686.00 5017.00 3000.00 31440.00 120000.00 20000.00 |

-6-

| Sl.No | Brief Description of Work | Quantity | Unit | Rate | Amount (Rs) |
| --- | --- | --- | --- | --- | --- |
| 10. 11. 12. 13. 14. 15. | Providing and fixing best country wood designed windows with wooden grill work including cost of fittings, polishing etc., complete. Providing plastering to wall surface in CM 1:6, 20mm thick including scaffolding, curing etc., Providing approved quality polished Granite slabs flooring/skirting, slabs set in CM 1:4, including curing etc., Providing granite tiled dadooing using 10mm thick flamed granite tiles.tiles set in C.M 1:4, including cost of materials, pointing, scaffolding etc., complete. Providing Decorative cornice works in Basement, art work in wall etc., as per design. Contingencies. and consultation charges. | 2.00 95.00 17.00 95.00 - - | Sqmt Sqmt Sqmt Sqmt - - | 10000.00 250.00 1800.00 1200.00 L.S L.S | 20000.00 23750.00 30600.00 114000.00 50000.00 26617.00 |

### Total Amount:          Rs. 5,00,000.00

(Rupees Five Lakhs  only.)